# GNS3 SSH Router Lab (Cisco IOS)

This lab demonstrates how to enable secure remote management using **SSH** between two Cisco routers in **GNS3**.

## Topology

R1 (Southampton) <----Gi1/0----> R2 (Portsmouth)

## IP Addressing

| Device | Interface | IP Address | Subnet Mask |

|-------|-----------|------------|------------|

| R1 (Southampton) | Gi1/0 | 192.168.1.2 | 255.255.255.0 |

| R2 (Portsmouth)  | Gi1/0 | 192.168.1.1 | 255.255.255.0 |

## Goals

- Configure hostnames and interfaces

- Verify connectivity with ping

- Enable SSH (domain name + RSA keys)

- Create local user authentication

- Restrict VTY access to SSH only

- Validate by SSHing from R2 → R1

## How to use

1. Build the topology in GNS3 (two routers connected via Gi1/0).

2. Copy/paste the configs from `configs/` into each router.

3. Verify:

- `show ip interface brief`

- `ping 192.168.1.1` and `ping 192.168.1.2`

- From R2: `ssh -l cybervault 192.168.1.2`

## Verification commands

```bash

show ip interface brief

show ip ssh

show run | section line\ vty